ESP32 CODE

/* terminal1 program

-----------------------------------------------------------------------------

* Pin layout should be as follows:

* Signal     Pin              Pin               Pin

*            Arduino Uno      Arduino Mega      MFRC522 board

* ------------------------------------------------------------

* Reset      9                5                 RST

* SPI SS     10               53                SDA

* SPI MOSI   11               51                MOSI

* SPI MISO   12               50                MISO

* SPI SCK    13               52                SCK

* voltage 3.3v

*/

// gsm module is connected with pin number 7 and pin number 8 of the arduino.

#include <SPI.h>

#include <MFRC522.h>

#include <LiquidCrystal.h>

#include <SoftwareSerial.h>

SoftwareSerial SIM900(7, 8); // gsm module connected here

String textForSMS;

#define SS_PIN 10

#define RST_PIN 9

MFRC522 mfrc522(SS_PIN, RST_PIN);        // Create MFRC522 instance.

// lcd pins

#define rs A1

#define en A2

#define d4 6

#define d5 5

#define d6 4

#define d7 3

// initialize the library with the numbers of the interface pins

LiquidCrystal lcd(rs, en, d4, d5, d6, d7);

// parents numbers

String f1001 = "+918220704543"; // student1 father cell phone number

String f1002 = "+918220704543";

String f1003 = "+918220704543";

void setup() {

Serial.begin(9600);        // Nodemcu is connected over here

randomSeed(analogRead(0));

SIM900.begin(9600); // original 19200. while enter 9600 for sim900A

SPI.begin();                // Init SPI bus

mfrc522.PCD_Init();        // Init MFRC522 card

//Serial.println("Scan a MIFARE Classic PICC to demonstrate Value Blocks.");

// set up the LCD's number of columns and rows:

lcd.begin(16, 2);

// Print a message to the LCD.

lcd.print("rfid based load moniter");

delay(3000);

lcd.clear();

lcd.print("Swip Your Card");

delay(1000);

}

void loop() {

// Prepare key - all keys are set to FFFFFFFFFFFFh at chip delivery from the factory.

MFRC522::MIFARE_Key key;

for (byte i = 0; i < 6; i++) {

key.keyByte[i] = 0xFF;

}

// Look for new cards

if ( ! mfrc522.PICC_IsNewCardPresent()) {

return;

}

// Select one of the cards

if ( ! mfrc522.PICC_ReadCardSerial()) {

return;

}

// Now a card is selected. The UID and SAK is in mfrc522.uid.

// Dump UID

Serial.print("Card UID:");

for (byte i = 0; i < mfrc522.uid.size; i++) {

//   Serial.print(mfrc522.uid.uidByte[i] < 0x10 ? " 0" : " ");

//   Serial.print(mfrc522.uid.uidByte[i], DEC);

}

Serial.println();

// Dump PICC type

byte piccType = mfrc522.PICC_GetType(mfrc522.uid.sak);

//    Serial.print("PICC type: ");

//Serial.println(mfrc522.PICC_GetTypeName(piccType));

if (        piccType != MFRC522::PICC_TYPE_MIFARE_MINI

&&        piccType != MFRC522::PICC_TYPE_MIFARE_1K

&&        piccType != MFRC522::PICC_TYPE_MIFARE_4K) {

//Serial.println("This sample only works with MIFARE Classic cards.");

return;

}

// defining Cards here

// student1

if( (mfrc522.uid.uidByte[0] == 57) && (mfrc522.uid.uidByte[1] == 240) && (mfrc522.uid.uidByte[2] == 01) && (mfrc522.uid.uidByte[3] == 04) ) // student1 card

{

lcd.clear();

lcd.print("scanning vehicle");

delay(500);

Serial.println("student 1");

// for gsm

sendsms("your vehicle over load fine RS100", f1001);

delay(1000);

lcd.setCursor(0, 1);

lcd.print("message sent");

delay(2000);

lcd.clear();

lcd.print("Swip Your Card");

delay(1000);

}

// student2

if( (mfrc522.uid.uidByte[0] == 26) && (mfrc522.uid.uidByte[1] == 141) && (mfrc522.uid.uidByte[2] == 07) && (mfrc522.uid.uidByte[3] == 177) ) // student2 card

{

lcd.clear();

lcd.print("AttendanceMarked");

delay(500);

Serial.println("student 2");

// for gsm

sendsms("pavithra/BCA/3YR is Present", f1002);

delay(1000);

lcd.setCursor(0, 1);

lcd.print("message sent");

delay(2000);

lcd.clear();

lcd.print("Swip Your Card");

delay(1000);

}

else

Serial.println("unregistered user");

}

void sendsms(String message, String number)

{

String mnumber = "AT + CMGS = \""+number+"\"";

SIM900.print("AT+CMGF=1\r");

delay(1000);

SIM900.println(mnumber);  // recipient's mobile number, in international format

delay(1000);

SIM900.println(message);                         // message to send

delay(1000);

SIM900.println((char)26);                        // End AT command with a ^Z, ASCII code 26

delay(1000);

SIM900.println();

delay(100);                                     // give module time to send SMS

// SIM900power();

}